FUNHOUSE

OBJECT: Take control of the FUNHOUSE from Rudy, the Dummy, by putting him to SLEEP. Put
Rudy to SLEEP by advancing the FUNHOUSE clock to MIDNIGHT. Shots that advance the clock are
the Jet Bumpers, the Center Ramp, and the Wind Tunnel.

MULTI-BALL: At 11:30 on the clock a LOCK is enabled, LOCK a ball and the clock will advance to
11:45. LOCK the next ball and the clock will advance to 12:00 putting Rudy to SLEEP. While Rudy is
sleeping shoot a ball into his OPEN MOUTH to start Multi-Ball.

MILLION PLUS: While in Multi-Ball shoot the ball into the TRAP-DOOR for 2 MILLION. The
TRAP-DOOR will close. Shoot a ball around the CENTER RAMP, to RE-OPEN the TRAP-DOOR for
3 MILLION, 4 MILLION etc.

MYSTERY MIRROR: Hitting Rudy in the JAW lights the MIRROR VALUE LIGHT. Shooting into the
WIND TUNNEL collects the value on the mirror. The value is changed by the JET BUMPERS.

LEFT BALL SHOOTER: The FUNHOUSE STEPS are lit by entering the MANHOLE to the Right of
the JET BUMPERS. Making the CENTER RAMP will divert the ball to the left ball shooter. Shoot for
one of the FLASHING SKILL SHOT values.

SKILL SHOT: From the right plunger shoot the ball BEHIND Rudy’s head to collect Rudy’s
HIDEOUT BONUS.

16-50003-1

FUNHOUSE

OBJET: Faites dormir ROBERT l’ Automate pour prendre possession de la DROLE DE MAISON.
Pour le faire, avancez les aiguilles de la pendule jusqu’ à Minuit par les Bumpeurs, la Rampe
Centrale et le Wind Tunnel (Passage sous le Miroir).

MULTI-BILLE: A 23 h 30 le Blocage “LOCK” est prêt. Bloquez une bille pour avancer la pendule sur
23 h 45, puis bloquez une seconde bille pour faire sonner Minuit et mettre Robert au lit. Quand
Robert dormira, envoyez la bille dans sa Bouche (Ouverte) pour commencer le Jeu en Multi-Bille.

MILLION PLUS: En jeu Multi-Bille, envoyez la bille dans la TRAPPE pour la fermer et avoir
2 MILLIONS, puis faites un passage sur la Rampe Centrale pour rouvrir la Trappe et jouer pour
3 MIOLLIONS, 4 MILLIONS etc.

MIRROIR MYSTERIEUX: Frappez la Mâchoire de Robert pour allumer une des VALEURS du MIROIR,
puis envoyez la bille dans le TUNNEL “WIND” (Passage sous ce Miroir) pour prendre cette valeur, Les
valeurs changent par les Bumpeurs.

LANCE BILLE GAUCHE: Envoyez la bille dans le TROU DROIT (sous les Bumpeurs) pour allumer al
Fleche “STEPS” au seuil de la Rampe Centrale. Si vous passez sur cette Rampe, le Déviateur
enverra la bille vers le LANCE BILLE GAUCHE. Avec ce Lance Bille essayez d’ obtenir une des
valeurs clignotantes de la Rampe “STEPS” (Rampe gauche).

LANCE BILLE DROIT: Envoyez la bille derrière la Tête de Robert pour avoir le BONUS ROBERT.

16-50003-1-Fr

|  | FUNHOUSE Switch Matrix |  |  |  |  |  |  | White |  | • • | Green |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dedicated Grounded Switches |  | Column Row | 1 Green- Brown J206-1 U20-18 | 2 Green- Red J206-2 U20-17 | 3 Green- Orange J206-3 U20-16 | 4 Green- Yellow J206-4 U20-15 | 5 Green- Black J206-5 U20-14 | 6 Green- Blue J206-6 U20-13 | 7 Green- Violet J206-7 U20-12 | 8 Green- Gray J206-9 U20-11 |  |
|  | Orange-Brown J205-1 Left Coin Chute (1) D1 |  | White- Brown J208-1 U18-11 1 | (9) Right Flipper 11 | (17) Slam Tilt 21 | (25) S-T-E-P "P" (Lower Right) 31 | (33) Left Slingshot (Kicker) 41 | (41) Dummy Jaw (Opto) 51 | (49) Rt Inside Flipper Return Lane 61 | (57) Rt Outside Flipper Return Lane 71 | (65) Not Used 81 |  |
|  | Orange-Red J205-2 Center Coin Chute (2) D2 |  | White- Red J208-2 U18-9 2 | (10) Left Flipper 12 | (18) Front Door 22 | (26) Top Superdog Standup Target 32 | (34) Left Flipper Return Lane 42 | (42) Right Outlane 52 | (50) Right Ball- shooter 62 | (58) Left Trough 72 | (66) Not Used 82 |  |
|  | Orange-Black J205-3 Right Coin Chute (3) D3 |  | White- Orange J208-3 U18-5 3 | (11) Start Button 13 | (19) Ticket Dispenser 23 | (27) Upper Left Gangway Rollunder 33 | (35) Left Outlane 43 | (43) Right Slingshot (Kicker) 53 | (51) Right Trough 63 | (59) Outhole 73 | (67) Not Used 83 |  |
|  | Orange-Yellow J205-4 4th Coin Chute (4) D4 |  | White- Yellow J208-4 U18-7 4 | (12) Plumb Bob Tilt 14 | (20) Test Position, Always Closed 24 | (28) Bottom Superdog Standup Target 34 | (36) Wind Tunnel Hole 44 | (44) S-T-E-P "T" (Upper Left) 54 | (52) S-T-E-P "E" (Upper Center) 64 | (60) Center Trough 74 | (68) Not Used 84 |  |
|  | Orange-Green J205-6 (5) D5 Normal Test Function Function Service Escape Credits Orange-Blue J205-7 |  | White- Green J208-5 U19-11 5 | (13) Steps Lights Frenzy 15 | (21) Lock Mech Right 25 | (29) Steps Track Lower 35 | (37) Trap Door 45 | (45) Steps Superdog (opto) 55 | (53) Dummy Eject Hole 65 | (61) Upper Right Loop Switch 75 | (69) Not Used 85 |  |
|  | Normal Test Function Function Volume Down Down (6) D6 |  | White- Blue J208-7 U19-9 6 | (14) Upper Ramp Switch 16 | (22) Steps Lights Extra Ball 26 | (30) Steps 500,000 36 | (38) Rudys Hideout Kickbig 46 | (46) Ramp Entrance 56 | (54) Upper Right Gangway Lane 66 | (62) Trap Door Closed 76 | (70) Not Used 86 |  |
|  | Orange-Violet J205-8 (7) D7 Normal Test Function Function Volume Up Up |  | White- Violet J208-8 U19-5 7 | (15) S-T-E-P "S" (Lower Left) 17 | (23) Lock Mech Center 27 | (31) Center Superdog Standup Target 37 | (39) Left Ball- shooter 47 | (47) Jet Bumper Lane 57 | (55) Lower Right Drop Hole 67 | (63) Upper Right Jet Bumper 77 | (71) Not Used 87 |  |
|  | Orange-Gray J205-9 (8) D8 Normal Test Function Function Begin Enter Test |  | White- Gray J208-7 U19-7 8 | (16) Upper Left Jet Bumper 18 | (24) Lock Mech Left 28 | (32) Steps Track Upper 38 | (40) Ramp Exit Track 48 | (48) Tunnel Kickout 58 | (56) Lower Jet Bumper 68 | (64) Not Used 78 | (72) Not Used 88 |  |
|  |  |  |  |  |  |  |  |  |  | 16-9178-1 |  |  |

|  |  | FUNHOUSE Lamp Matrix |  |  |  |  | Yellow (B+) |  | Red |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Column Row | 1 Yellow- Brown J137-1 Q98 | 2 Yellow- Red J137-2 Q97 | 3 Yellow- Orange J137-3 Q96 | 4 Yellow- Black J137-4 Q95 | 5 Yellow- Green J137-5 Q94 | 6 Yellow- Blue J137-6 Q93 | 7 Yellow- Violet J137-7 Q92 | 8 Yellow- Gray J137-9 Q91 |  |  |
|  |  | Red- Brown J133-1 Q90 1 | (1) Gangway 75,000 (Left) 11 | (9) Clock 45 Minutes 21 | (17) Clock 40 Minutes 31 | (25) Clock 9 o’ clock 41 | (33) Lower Left Jet Bumper 51 | Left & (41) Inside Rt Flipper Lanes (x 2) 61 | Magic (49) Mirror Lights Extra Ball 71 | (57) Million Plus 81 |  |  |
|  |  | 2 Red- Black J133-2 Q89 | (2) Gangway 100,000 12 | (10) Clock 8 o’ clock 22 | (18) Clock 35 Minutes 32 | (26) Clock 7 o’ clock 42 | (34) Upper Left Jet Bumper 52 | (42) S-T-E-P "S" 62 | (50) Upper Right Jet Bumper 72 | (58) Special Outlanes (x 2) 82 (59) Trap Door Frenzy 83 |  |  |
|  |  | Red- Orange J133-4 Q88 3 | (3) Gangway 150,000 13 | (11) Clock 6 o’ clock 23 | (19) Clock 30 Minutes 33 | (27) Clock 5 o’ clock 43 | (35) Superdog Lamp (x 2) 53 | (43) Trap Door Bonus 63 | (51) S-T-E-P "P" 73 |  |  |  |
|  |  | Red- Yellow J133-5 Q87 4 | (4) Gangway 200,000 14 | (12) Clock 25 Minutes 24 | (20) Clock 20 Minutes 34 | (28) Clock 4 o’ clock 44 | (36) Steps Lights Frenzy 54 | (44) Ramp Scores 250,000 64 | Magic (52) Mirror Lights Million 74 | (60) Ramp "Steps" Lamp 84 |  |  |
|  |  | 5 Red- Green J133-6 Q86 Red- Blue J133-7 Q85 6 | (5) Gangway 250,000 15 | (13) Clock 15 Minutes 25 | (21) Clock 3 o’ clock 35 | (29) Clock 2 o’ clock 45 | (37) Steps Lights Extra Ball 55 | (45) S-T-E-P "T" 65 | Magic (53) Mirror Lights Jet Bumpers 75 | (61) Magic Mirror Arrow 85 |  |  |
|  |  |  | (6) Gangway Lights Extra Ball 16 | (14) Clock 10 Minutes 26 | (22) Clock 1 o’ clock 36 | (30) Clock 5 Minutes 46 | (38) Steps 500,000 56 | Upper (46) Left Gangway Lane 66 | Magic (54) Mirror Lights Superdog 76 | (62) S-T-E-P "E" 86 |  |  |
|  |  | 7 Red- Violet J133-8 Q84 | (7) Shoot Again 17 | (15) Clock 12 Minutes 27 | (23) Clock 11 o’ clock 37 | (31) Clock 55 Minutes 47 | (39) Ramp Overhead Lamp 57 | (47) Extra Ball Lamp 67 | Magic (55) Mirror Opens Gate 77 | (63) Million 87 |  |  |
|  |  | Red- Gray J133-9 Q83 8 | (8) Steps Open (Gate) 18 | (16) Clock 0 Minutes 28 | (24) Clock 50 Minutes 38 | (32) Clock 10 o’ clock 48 | Right (40) Gangway Overhead Lamp 58 | (48) Lock 68 | Magic (56) Mirror Lights Quick Multi-ball 78 | (64) Start Button (Cabinet) 88 |  |  |
|  |  |  |  |  |  |  |  |  | 16-9178-2 |  |  |  |

Fonts used: Helvetica

Cards status:

16-50003-1 confirmed.

16-50003-1-Fr confirmed.

16-9178-1 Switch-Matrix confirmed.

16-9178-2 Lamp-Matrix confirmed.

16-9182-1 Fuse card is confirmed, available in a separate file called Williams_WPC_Fuse_cards in the Williams section.

If you have any other information about these cards, please send a readable picture or scan to me.

Enjoy and have fun,

Peter

www.inkochnito.nl

If you like my work, please send me a donation via Paypal.